PCB Design Project

USB LED Keychain

A Decorative Yet Functional Electronic Accessory

Version 1.0  |  PCB Design Documentation

# 1. Project Overview

The USB LED Keychain is a compact, creative PCB design project that serves a dual purpose: it looks like an ordinary decorative keychain when not in use, but springs to life with glowing LEDs the moment it is plugged into any USB port. The design is intentionally minimal, using just a resistor and one or more LEDs to create a satisfying light-up effect powered directly from the 5 V USB bus.

The project is ideal for beginners learning PCB design, makers who want a quick weekend build, or anyone looking for a personalized, handcrafted gadget. No firmware, no microcontroller, no drivers required — plug it in and it glows.

# 2. Concept & Design Philosophy

## 2.1 The Idea

Most electronic keychains are purely passive accessories. This project flips that idea: the PCB is shaped and decorated to look attractive when hanging from a bag or set of keys, but contains a hidden function. The USB-A connector on the edge acts both as the mechanical mounting point of the keychain loop and as the functional power interface.

## 2.2 Design Goals

- Keep the circuit as simple as possible (resistor + LED only).

- Make the PCB itself visually appealing — use silkscreen art, custom shapes, or colour soldermask.

- Ensure reliable operation from any USB 2.0 / 3.0 / 3.1 host or charger (5 V, ≥ 100 mA).

- Keep the BOM cost under ₹50 per unit.

- Make it easy to assemble by hand, even without soldering experience.

# 3. How It Works

The circuit is elegantly straightforward. When the USB-A plug is inserted into a laptop, PC, power bank, or charger, the host supplies 5 V on VBUS (pin 1) with GND on pin 4. The data lines (D+, D−) are left unconnected since no data communication is needed.

The power path is simply:

VBUS (5 V)  →  Current-Limiting Resistor  →  LED(s)  →  GND

The resistor limits the current flowing through the LED to a safe level (typically 10–20 mA), preventing it from burning out. The LED converts this current into visible light. That is the entire circuit.

# 4. Circuit Design

## 4.1 Schematic Components

| Ref. | Component | Value / Part | Purpose |
| --- | --- | --- | --- |
| J1 | USB-A Connector | USB Type-A Male (through-hole) | Power input & mechanical anchor |
| R1 | Resistor | 100 Ω – 220 Ω, 1/4 W | Limits LED current to safe level |
| D1 | LED | 3 mm or 5 mm, any colour | Emits light when USB is connected |
| (opt.) D2–D4 | Additional LEDs | Same as D1, wired in parallel | Multi-LED lighting effect |

## 4.2 Resistor Value Calculation

Use Ohm's Law to find the correct resistor value:

R = (V_supply − V_forward) ÷ I_LED

Example for a red LED (V_f ≈ 2.0 V) at 15 mA:

R = (5.0 − 2.0) ÷ 0.015 = 200 Ω  →  use 220 Ω (nearest standard value)

| LED Colour | Typical V_forward | Recommended R (15 mA) | Recommended R (10 mA) |
| --- | --- | --- | --- |
| Red | 1.8 – 2.2 V | 187 Ω → 180 Ω | 280 Ω → 270 Ω |
| Green | 2.0 – 2.4 V | 173 Ω → 180 Ω | 260 Ω → 270 Ω |
| Blue / White | 2.8 – 3.5 V | 100 Ω – 133 Ω | 150 Ω – 200 Ω |
| Yellow / Orange | 1.9 – 2.2 V | 187 Ω → 180 Ω | 280 Ω → 270 Ω |

# 5. PCB Layout & Physical Design

## 5.1 Board Shape & Dimensions

The PCB is designed in a custom decorative shape — for example a star, lightning bolt, heart, shield, or cartoon character — using the Edge.Cuts layer in KiCad or the Board Outline layer in EasyEDA. Typical dimensions are 40 mm × 30 mm, keeping the board light enough to hang on a keychain without weighing it down.

## 5.2 Layer Stack-up

- 2-layer PCB (top copper + bottom copper): sufficient for this simple design.

- Soldermask colour: your choice — black, blue, red, green, purple, or gold are popular options.

- Silkscreen (top): used for decorative art, project name, and component reference designators.

- Silkscreen (bottom): optional — can include a QR code, social handle, or additional art.

## 5.3 Component Placement

- USB-A connector: placed at the top edge of the board so the plug protrudes naturally for insertion.

- Resistor R1: placed close to the VBUS pad, oriented horizontally for a neat appearance.

- LED(s): spread across the board face to maximise visual effect; position them near the silkscreen art for a glowing outline or accent effect.

- Keychain hole: a 3 mm drill hole in the corner of the PCB, reinforced with a copper ring (annular ring) and covered by soldermask opening for durability.

## 5.4 Trace Routing

- Use 0.5 mm – 1.0 mm trace width for power lines (VBUS and GND) — more than adequate for ≤ 60 mA.

- Keep traces short and direct; aesthetics count here since the PCB is a display item.

- Fill unused board area with a GND copper pour to improve aesthetics and reduce etching time if home-fabricating.

# 6. Tools & Software Used

| Tool | Purpose | Notes |
| --- | --- | --- |
| KiCad 7 / EasyEDA | Schematic capture & PCB layout | Both are free and beginner-friendly |
| JLCPCB / PCBWay | PCB fabrication | 5 pcs from ≈ $2 USD with shipping |
| Soldering iron (25–40 W) | Component assembly | Through-hole parts are easy to solder |
| Multimeter | Continuity & voltage testing | Verify before first USB insertion |

# 7. Assembly Instructions

Follow these steps after receiving the fabricated PCB:

- Step 1 — Inspect the PCB: Check for any manufacturing defects, short circuits on the copper pours, or missing drill holes.

- Step 2 — Solder R1 first: Insert the resistor into its through-hole footprint, bend the leads, solder from the bottom, and trim excess lead.

- Step 3 — Solder the LED(s): Observe correct polarity — the longer lead (anode, +) goes into the square pad. Solder and trim.

- Step 4 — Solder the USB-A connector: Align the four pins and two mounting tabs, tack one corner, verify alignment, then solder all pins.

- Step 5 — Visual inspection: Under good lighting, look for solder bridges, cold joints, or missing connections.

- Step 6 — Continuity check: Use a multimeter to confirm VBUS is not shorted to GND before plugging in.

- Step 7 — Power-up test: Plug into a USB port. The LED(s) should illuminate immediately. If not, re-check polarity and solder joints.

- Step 8 — Attach keychain ring: Thread a split ring or lanyard through the 3 mm hole and enjoy your glowing keychain!

# 8. Bill of Materials (BOM)

| # | Component | Specification | Qty | Est. Cost | Source |
| --- | --- | --- | --- | --- | --- |
| 1 | PCB (fabricated) | Custom shape, 2L, any colour | 1 | ₹18–₹25 | JLCPCB / PCBWay |
| 2 | USB-A Male Connector (TH) | Through-hole, 4-pin | 1 | ₹3–₹5 | Local / Robu.in |
| 3 | Resistor | 100–220 Ω, 1/4 W, through-hole | 1 | ₹0.50 | Any electronics shop |
| 4 | LED | 3 mm or 5 mm, colour of choice | 1–4 | ₹1–₹3 each | Any electronics shop |
| 5 | Keychain Split Ring | 25–30 mm diameter | 1 | ₹2 | Stationery / craft store |
|  | TOTAL ESTIMATED COST |  |  | ₹25–₹40 |  |

# 9. Design Variations & Upgrades

## 9.1 Multiple LEDs

Wire two, three, or four LEDs in parallel (each with its own resistor) to create a dazzling multi-light effect. Each LED gets an individual resistor to ensure even current distribution.

## 9.2 RGB LED

Replace the single-colour LED with a common-cathode RGB LED and use three separate resistors — one per colour channel. All three channels will light simultaneously for white light, or you can leave one or two channels disconnected for custom colour mixing.

## 9.3 Custom PCB Shapes

Popular decorative PCB shapes for this project include:

- Lightning bolt — perfect for a hacker or tech theme.

- Star or moon — great for gifting.

- Heart — a popular Valentine's Day project.

- Custom logo or initials — personalised using silkscreen art.

- Game controller, spaceship, or other icons — limited only by imagination.

## 9.4 Blinky Circuit (Optional Upgrade)

For an advanced version, replace the simple resistor with a 555 timer IC in astable mode wired to the LED. This makes the LED blink at a configurable frequency, drawing more attention and making the keychain even more eye-catching.

# 10. Safety & Best Practices

- Always verify there is no short between VBUS and GND before inserting into a computer or charger — use a multimeter in continuity mode.

- Use a USB hub or phone charger for initial testing if you are concerned about computer safety.

- The circuit draws very little current (< 60 mA for 4 LEDs), well within USB limits (500 mA for USB 2.0).

- Ensure the LED polarity is correct before powering — a reverse-biased LED simply will not light up, but reverse voltage can damage some LED types.

- Keep solder joints clean and free of flux residue; apply flux cleaner if needed for a professional finish.

# 11. Learning Outcomes

Completing this project teaches the following skills:

| Skill Area | What You Learn |
| --- | --- |
| PCB Design | Schematic entry, footprint placement, trace routing, Gerber file export |
| Electronics Basics | Ohm's Law, LED characteristics, current limiting, USB pinout |
| Soldering | Through-hole soldering technique, connector mounting, lead trimming |
| Testing & Debug | Multimeter use, continuity checks, fault diagnosis |
| Creative Design | PCB art, silkscreen layout, custom board shapes, colour selection |

USB LED Keychain — PCB Design Project  |  Simple. Elegant. Functional.

"A decorative keychain that glows to life the moment it meets a USB port."